PISO Shift Register w/ LEDS

Materials:

- 74HC165 shift register

- 5V power supply

- 9x pushbuttons

- 9x 1 kΩ resistors

- 2x 470 Ω resistors

- 1x 10 kΩ resistor

- 1x 100 kΩ resistor

- 2 LEDs:

  - 1 Green, 1 Red

- 555 timer

- 1x 0.1 µF capacitor

- 1x 1 µF capacitor

How it works:

This circuit uses a 74HC165 parallel-in serial-out (PISO) shift register to read multiple pushbutton inputs and convert them into a single serial data stream. The pushbuttons are connected to the parallel input pins of the shift register, with 1 kΩ resistors used as pull-down resistors to ensure stable, defined logic levels when the buttons are not pressed. When a button is pressed, it sends a HIGH signal to its corresponding input pin.

The 555 timer is configured as an astable oscillator to generate a continuous clock signal. This clock signal is fed into the shift register, controlling when data is shifted out. When the load pin is activated, the current states of all pushbuttons are captured simultaneously into the register. Then, with each clock pulse from the 555 timer, the stored data is shifted out one bit at a time through the serial output pin.

The LEDs are connected to indicate system activity and output status. For example, one LED may blink according to the clock signal, showing that shifting is occurring, while the other LED may reflect the serial data output. The 470 Ω resistors limit current to protect the LEDs.

The capacitors and resistors connected to the 555 timer determine the frequency of the clock signal, while the 0.1 µF capacitor also helps filter noise and stabilize operation.